Aufruf:   Z80ASM [COMMAND][,COMMAND]

COMMAND:   <fname>[.<DRIVE>][/<OPTION>]

<DRIVE>

first Letter:  .Z80 Source

second Letter:  Destination

thir� Lette�:   Listin�

Letter equ:

@ default drive

X Console

Y LST: Device

ڠn� Listing

<option>

A:   absolute Mode  .COM file

C:   enable Output to Console

D:   don't do lower to UPPER case Conversion

E:   execute lower to UPPER case conversion

F:   full Listing (2 passes)

H:   .HEX File output

I:   special indirect Command File  .SUB

K:   kill all Console I/O and detach

L:   list output partial (1 pass)

M:   Microsoft REL Format

N:   new OPTION byte

P:   enable output to the LST: Device

Q:   Quit

R:   Standard REL File

S:   Symbol Table

U:   declare undefined symbols as external

X:   Cross Refference

Y:   disable Cross Refference

6:   M rel  6 significant

7:   M rel  7 significant

T:   input Time and Date   16 Zeichen bis , oder <CR>

^C   abord compilation

^S   Start scrolling

^Q   stop  scrolling

^Z   console driver

^P   list driver

?    current Filename and Line be assembled

.ACCEPT [STRING,]<SYMBOL>          .accept DEBUG_FLAG

DEFC

DC

DEFZ

Labels    $ ? . @ 0..9 A..Z a..z _ %    16 significant