BSTAM from L. P. Enterprises

----------------------------

The BSTAM system distributed as TRANSMIT.COM and

RECEIVE.COM have been installed with interfaces

to specific UART/USART chips. The system for

S100 systems is supplied with routines for an

8251 USART addressed at ports 4/5. This is usable

without modification with the IMSAI 2SIO PORT B or

the North Star Horizon Right Serial Port. The source

of the routine is the file UHOR.ASM.

Other files are supplied with the system as further

examples of interfacing. The user should familiarize

himself with the I/O requirements of the system to

be equipped and edit and assemble an appropriate

I/O interface module. This is installed thus:

A> ASM UART	assemble UART.ASM, your own

interface

A> DDT RECEIVE.COM

I UART.HEX

R		read UART.HEX over RECEIVE.COM

^C		boot

A> SAVE 20 RECEIVE2.COM

save the patched file with a

temporary name for testing

A> DDT TRANSMIT.COM

I UART.HEX

R

^C		same again for transmit

A> SAVE 27 TRANSMI2.COM

The files RECEIVE2 and TRANSMI2 will now be customized

to your own I/O patches, and can be tested.

Don't forget that if two computers are taking to each other

along an RS232 cable set, one must appear as a data terminal

to the other. This normally involves reversing the leads

between pins 2 and 3 of one of the EIA connectors.

Machine 1		Machine 2

---------		---------

PIN 2       to	  PIN 3

PIN 3       to	  PIN 2

PIN 7       to          PIN 7

No other handshaking is required, since all of the acknowlegement

is performed in software.